UnitControl 2.5 (29.04.1996)

==============================

The  Workbench  tool  UnitControl  allows  you to carry out device specific

settings  for  each  individual  device connected to the SCSI bus.  It also

allows   you   to  control  removable  media  and  tape  streamers.   Start

UnitControl  by  double-clicking  on  the  icon.  The following main window

appears:

The  upper  part  of the window displays the units present on the SCSI bus.

You  can  select  these  for further actions by clicking on the units.  The

title  of  the  selected  unit then changes color.  If you should have more

units than will fit in the window then you can scroll through the list with

the scroll bar on the right-hand side of the display window.

--- Quit

As  long  as  none of the devices is selected, only the Quit button will be

available  from  the five buttons in the lower part of the main window.  By

clicking on this button you will exit UnitControl.

As  soon  as you select a device the other buttons become available.  These

buttons allow you to control various actions of the connected device.

--- Eject:

If  the  SCSI  device  supports software controlled ejection of the medium,

then  this  button will eject the medium.  For devices which do not support

ejection of mediums the activation of this button normally causes the drive

motor to stop.

--- Stop:

This  button  turns off the drive motor of the device selected.  Hard disks

or  removable  drives  which you only use sporadically can be turned off in

this way to avoid unnecessary wear and tear and power consumption.

--- Start:

A device deactivated with Stop or for other reasons can be reactivated by

using this button.

--- Rescan

Rescan SCSI Bus for new SCSI units.

--- Mount

Try to mount partitions from a unit.

--- Prefs

Allows to set a Resident Tag with 'Save'.  May speedup your next reboot.

--- x x x x x x x x  Unit Scan Array

The unit which is activated here is scanned during the boot phase.

--- SCSI Reset Delay

The seconds the device waits after a SCSI Reset.

--- Inquiry Error Delay

The seconds the device waits after an Inquiry Error

--- Inquiry Error Retry

The retry count for an Inquiry after an Error.

--- Error Retrys

The retry count for an SCSI Operation after an SCSI Error.

--- Selection Timeout

The count of chip ticks the chip uses to determine that a selection failed.

--- Lun Reselection

Using Reselection between single Luns of a Unit.

I know no Unit which supports this.

--- Options:

By activating this button a new window is opened, where you can set special

device  parameters  for the unit selected.  This tool can be very useful if

problems occur in transmitting data to a certain device without other units

being   affected.    The   settings  under  Special  also  allow  the  data

transmission to be adjusted to maximum speed.

You  should  have  various information about the selected device at hand so

that  you  can  set  the  options  correctly.  In general, you can find the

information  in  the  datasheets  or  the  operating  instructions  of  the

connected device.

--- Asynchron/Synchron

This  button  switches  between  synchronous and asynchronous access to the

device.   The  default setting is determined from the information contained

in  the  RDB.   In  synchronous  mode  a  certain  number of data words are

transferred  in  a block, followed by a handshake.  This mode allows higher

transmission   rates  if  the  device  supports  them.   Unfortunately  the

likelihood  of  errors in the transmission also increases, in proportion to

the  size  of  the  block  to be transmitted.  In synchronous mode up to 10

MByte  data  transmission  rate is possible.  The maximum transmission rate

for a particular device is, however, affected by both the device itself and

the length and type of the SCSI cable used.

When  you activate the synchronous mode, the SCSI Controller and the device

concerned  automatically  negotiate the maximum transmission rate.  This is

mostly  10  Mb/sec for FAST SCSI devices, and 4 or 5 Mb/sec for normal SCSI

or SCSI-II devices.

If  10 Mb/sec is automatically set this does not mean that the transmission

can  take  place  securely  at  this  rate  when  the  synchronous  mode is

activated.  If occasional transmission errors occur or the hard disk "hangs

up"  at  high  transmission  rates  after setting the synchronous operation

mode,  this  can  be an indication that the connection to the device is not

secure  enough  for  the  chosen  speed.   This  problem occurs because the

transmission  rates are extremely dependant on the cable length.  In such a

case you should set the rate lower to prevent transmission faults.

Experiments  have  shown  that the highest setting for the synchronous data

transmission   is   not   necessarily   the  fastest.   Today,  nearly  all

manufacturers  rate  the  speed  of  their  hard  disks at 10 Mb/sec, which

relates  to the SCSI bus speed.  In reality, today (Autumn 1993) hardly any

hard  disks  transfer  more than 4 Mb/secec as usable sustained output.  In

practice, with a fast drive in the gigabyte range, for example, it has been

shown  that  the  transfer  values  determined from the common test program

DiskSpeed  4.2 were faster in synchron-6MB-mode than in synchron-10MB-mode.

It  is  therefore  sensible  to try out various settings with the help of a

test program when determining the optimal setting for a certain drive.

--- Slider BYTES/HANDSHAKE

This  adjuster  allows you to set the number of bytes to be transmitted per

handshake.   The  optimal  value  for  this  setting can only be determined

through  trial  and  errors.   A  setting  which  is  too  high  can  cause

transmission  errors,  which  significantly lowers the speed or even causes

read  errors.   The  regulator  is set to 8 as a starting point, which will

provide  you  with  an  acceptable  transmission rate with an extremely low

likelihood of errors.

--- Slider SYNCHRON MB/S

Here  you  can manipulate the speed of the data transmission in synchronous

operation.   As  previously  explained,  the SCSI Controller and the device

concerned  negotiate  the  maximum  transmission  rate.  If you should have

problems  with the device connected, in the form of read errors or SCSI bus

hang-ups  despite  this,  then you can decrease this value to check whether

the  device  or  the connection (cable) are unable to correctly execute the

high transmission speed.  This problem can also occur if the device accepts

10  Mb/sec  as a permitted value because the transmission rate is extremely

dependant on the cable length.

Both the Bytes/Handshake and the Synchron Mb/sec adjusters should always be

considered as linked. To find the real best possible setting some test and

trial work is necessary. In general, the default values should provide

secure transmission at the optimum transmission rate.

--- Reselection

SCSI devices are able to hand over the bus during long continual operations

to  allow  other devices to transmit data.  At the end of the operation the

device  then  demands  the  bus once again to convey its own results.  This

process is called Reselection.  As not all devices support this process and

some  devices which do support it do not work correctly in this mode, it is

possible  to activate or deactivate this mode specifically for each device.

Reselection  has  practically  no effect in access to an individual device.

Reselection requires additional protocol so you should expect a slight loss

of performance.  Only when several devices are active simultaneously on the

bus is an increase in performance likely.

--- FWC Mode

Use this to enable/diable the write RAM cache.

--- Removable

Selects if a SCSI unit is a removable or not.

--- Quit

Quit allows you to exit the Special window, but it does not change anything

in the device configuration.

--- Set

Set allows you to save the settings made for the device.

--- Reset

This  resets  the SCSI bus.  If the SCSI bus should hang up (which you will

recognise  because  the  bus  activity  LED  on  one  or more of the drives

connected  will  be  lit  continuously)  then  you can return the bus to an

active state with this button.

Reset  affects  the  whole  bus,  not  only  the device selected.  If other

devices  are  transmitting  data  when this button is activated, it will be

lost.   This  can  vary  from  harmless  read  errors  to the loss of whole

partitions.   Use  reset only if there is no disk activity in the system or

as the very last resort if the SCSI bus has crashed .